Shreya Panchal

Assignment :- 7

Module -7: Network fundamental

1- Which of the following messages in the DHCP process are broadcasted? (Choose two)
Ans: A. Request 
         B. Offer 
         C. Discover 
         D. Acknowledge
Explanation:
DHCP Offer and DHCP Acknowledge messages are broadcast by the server so the client can receive them even before it has an IP address.
 
2- Which command would you use to ensure that an ACL does not block web-based TCP traffic? 
Ans: A. permit any 
         B. permit tcp any any eq 80 
         C. permit tcp any eq 80 
         D. permit any any eq tcp
Explanation:  
The command permit tcp any any eq 80 allows HTTP web traffic on TCP port 80 to pass through the ACL, ensuring web-based traffic is not blocked.

3-Explain Network Topologies 
Ans:
          Network Topology: Network topology defines the structure of a network how devices are connected and how data flows between them.
          Types of:  1. BUS Topology
                             2. STAR Topology
                             3. RING Topology
                             4. MESH Topology
                             5. TREE Topology
                             6. HYBRID Topology
       1. What is bus Topology ?
All computers are connected to one single main wire. If the main wire breaks, all computers stop working.
Advantages:
      1) Easy to install

2) Cheap and low cost

Disadvantages:

1) If main cable breaks, whole network stops

2) Slow when many computers are connected
          2. What is star topology?
All computers are connected to one central device (hub or switch). If one computer fails, others still work.

Advantages:
      1) Easy to manage

2) One computer failure does not affect others

Disadvantages:

1) Hub or switch failure stops network

2) Cost is high

3.What is ring topology?
Computers are connected in a round circle. Data moves one by one from computer to computer.
Advantages:
       1) No data collision

2) Equal chance for all computer

Disadvantages:
      1) One computer failure stops network

2) Hard to add or remove computers

4.What is mesh topology?
Each computer is connected to every other computer. Very safe and reliable, but costly.
Advantages:
      1) Very reliable network

2) High security

Disadvantages:
     1) Very expensive

2) Difficult to install
          5. What is tree topology?
Computers are connected in a tree shape (main line + branches). Used in big networks.
Advantages:
      1) Easy to manage large networks

2) Scalable

Disadvantages:
     1) Main cable failure affects many devices

2) Expensive
          6. What is hybrid topology?
It is a mix of two or more topologies (example: star + bus).
Advantages:
     1) Flexible

2) Scalable

Disadvantages:
    1) Expensive

2) Complex to design

4-Explain TCP/IP Networking Model 
Ans:
DEFINATION:
The TCP/IP model is a 4-layer networking model used for data communication over the Internet and most modern networks. It defines how data is packaged, addressed, transmitted, routed, and received.
1. Application Layer:
      Provides network services to user applications.
2. Transport Layer:
      Provides end-to-end communication between devices.
3. Internet Layer:
      Handles logical addressing and routing of packets.
4. Network Access Layer:
       Defines how data is physically transmitted over the network.

5-Explain LAN and WAN Network 
Ans:  Networks are commonly classified based on their geographical coverage. The two most important types are LAN (Local Area Network) and WAN (Wide Area Network).

1. LAN (Local Area Network)
Definition: A LAN is a network that covers a small geographic area, such as.
               1. A home
               2. An office
               3.A single building or campus
2. WAN (Wide Area Network)
Definition: A WAN covers a large geographic area, such as.
                1. Cities
                2. Countries
                3. Continents

6-Explain Operation of Switch 
Ans:  A network switch is a Layer 2 (Data Link layer) device used in LANs to connect multiple devices    and forward data efficiently based on MAC addresses.

         1. Basic Function of a Switch

A switch:

- Receives Ethernet frames

- Learns MAC addresses

- Forwards frames only to the correct destination port

- Reduces network collisions

2. MAC Address Learning

When a switch receives a frame:

- It reads the source MAC address

- Stores it in the MAC address table (CAM table)

- Associates the MAC address with the incoming port

This allows the switch to know where devices are located.

3. Frame Forwarding Process

a) Known Unicast

- If the destination MAC is in the MAC table:

  - The switch forwards the frame only to the specific port

- This improves performance and security

b) Unknown Unicast

- If the destination MAC is not in the MAC table:

  - The switch floods the frame to all ports except the source port

- Once the destination responds, the switch learns its MAC

c) Broadcast

- Frames sent to FF:FF:FF:FF:FF:FF

- Forwarded to all ports except the source

- Used by protocols like ARP and DHCP

4. Collision Domain and Broadcast Domain

- Each switch port is a separate collision domain

- All ports are in the same broadcast domain (unless VLANs are used)

5. Switching Methods

a) Store-and-Forward

- Entire frame is received first

- Error checking (CRC) is performed

- More reliable but slightly slower

b) Cut-Through

- Begins forwarding after reading destination MAC

- Faster but no error checking

6. Duplex Operation

- Switches support full-duplex

- Devices can send and receive simultaneously

- Eliminates collisions

7. VLAN Support (Advanced Operation)

- Switches can divide networks into Virtual LANs (VLANs)

- Improves security and reduces broadcast traffic

- Each VLAN is a separate broadcast domain

8. Example of Switch Operation

- PC1 sends a frame to PC2

- Switch learns PC1’s MAC on Port 1

- Looks up PC2’s MAC in the MAC table

- Forwards the frame only to PC2’s port

7-Describe the purpose and functions of various network devices
Ans:  Network devices are used to connect, manage, secure, and optimize communication between computers and networks. Each device has a specific role in data transmission. 

     1. Hub: Basic device to connect multiple computers in a LAN.
     2. Switch: Efficiently connects devices within a LAN.
     3. Router: Connects different networks together.
     4. Bridge: Divides a LAN into segments to reduce traffic.
     5. Modem: Connects a network to an ISP.
     6. Access Point (AP): Provides wireless connectivity.
     7. Firewall: Protects the network from unauthorized access.
     8. Repeater: Extends network distance.
     9. Gateway: Connects different network types or protocols.

7-Make list of the appropriate media, cables, ports, and connectors to 8
Ans: list of 8 common networking media, showing the cable type, port, and connector used for each.

            Appropriate Media, Cables, Ports, and Connectors (8):

1.Ethernet (LAN – Wired)
2. Fiber Optic (High-speed / Backbone)
3. Coaxial Cable
4. Telephone Network
5. Serial Communication
6. Console Connection (Network Devices)
7. USB Network Connection
8. Wireless (Wi-Fi)


8-connect switches to other 
Ans:  Switches can be connected together to expand a network and allow devices on different switches to communicate.

1. Using Ethernet Cable (Most Common)
       a) Straight-Through Cable
       b) Crossover Cable (Older Switches)
2. Using Fiber Optic Cable (Long Distance / High Speed)
3. Trunk Link (VLAN Networks)
4. Stacking Switches (Advanced)
5. Key Points to Remember (Exam Focus)

9-Define Network devices and hosts
Ans: Network Devices
Definition: Network devices are hardware components used to connect, control, manage, and secure communication between computers and networks.
Examples:

- Router

- Switch

- Hub

Hosts
Definition: A host is any device that is connected to a network and has an IP address, capable of sending or receiving data.
Examples:

- Desktop computer

- Laptop

- Smartphone